# XX September 2018, Susan M. O’Neill,  USDA Forest Service, Pacific Northwest Research Station, Pacific Wildland Fire Sciences Lab, 400 N. 34th St., Suite 201, Seattle, WA 98103, (206) 732-7851

# Air Quality Challenges of Prescribed Fire in Complex Terrain and the Wildland Urban Interface

# Colton W. Miller, University of Washington, School of Environmental and Forest Sciences, Box 352100, Seattle, WA 98195

Miriam L. Rorig, USDA Forest Service, Pacific Northwest Research Station, Pacific Wildland Fire Sciences Lab, 400 N. 34th St., Suite 201, Seattle, WA 98103

Susan M. O’Neill*, USDA Forest Service, Pacific Northwest Research Station, Pacific Wildland Fire Sciences Lab, 400 N. 34th St., Suite 201, Seattle, WA 98103

# Abstract

# Prescribed fires in forest ecosystems can negatively impact health and safety by transporting smoke downwind into nearby communities. This is known to occur around Bend, Oregon, USA, where burning in the wildland-urban interface on the Deschutes National Forest has resulted in smoke intrusions into populated areas. The number of suitable days for prescribed fires is limited due to the necessity for moderate weather conditions as well as wind directions that do not carry smoke into Bend. To better understand the conditions leading to these intrusions and to assess predictions of smoke dispersion from prescribed fires, we collected data from an array of weather and particulate monitors over the autumn of 2014 and spring of 2015. We characterized the observed winds to compare with meteorological and smoke dispersion models. The results from this study indicated that dispersion modeling can be useful for anticipating smoke intrusions, but significant errors in wind speed and direction of the meteorological models can lead to mischaracterizations of smoke intrusion events. Additionally, using higher resolution meteorological and dispersion models can improve the prediction of both timing and location of these events.

Key words: Prescribed fire, PM2.5, Smoke, Dispersion modeling

Introduction

Wildland fire is a naturally occurring ecological process, and many forests in the western United States (U.S.) were shaped by the periodic occurrence of fire (Covington and Moore 1994; Bowman et al. 2009). A history of timber harvesting, grazing, and fire suppression altered the role fire played on the landscape and, in some cases, caused shifts in forest structure and composition (e.g. Hessburg et al. 2005). These changes led to widespread fuel accumulations which can enhance the intensity and severity of wildfires (e.g. Graham et al. 2004). Forest managers use prescribed fire to improve forest health and create a diversity of plant and wildlife habitat (Covington et al. 1997; Agee et al. 2010). Additionally, prescribed fires reduce forest fuels and mitigate the intensity and effects of wildfires (Mitchell et al. 2009). However, planning prescribed fires presents challenges in avoiding undesired consequences to communities such as smoke and visibility.

Exposure to the smoke generated from forest fires (wild and prescribed) have negative health impacts (Delfino et al. 2009; Morgan et al. 2010; Rappold et al. 2011),  and the associated costs are substantial (Kochi et al. 2010). Particles smaller than 2.5 micrometers in aerodynamic diameter (PM2.5) is the pollutant of most concern within smoke, and the U.S. Environmental Protection Agency (EPA), through the Clean Air Act of 1970, has set standards as defined by the National Ambient Air Quality Standards (NAAQS) to protect the public. The maximum allowable level of PM2.5 averaged over a 24-hr period is 35 g/m3. States may also implement their own regulations in addition to the NAAQS specified by EPA.

Forest management agencies are increasingly moving away from fire exclusion and towards policies that balance modified suppression with the use of prescribed fire to achieve multiple ecological objectives (Miyanishi 2001). Therefore, predicting smoke impacts and, probably more importantly, determining meteorological conditions that reduce smoke impacts from prescribed fires, is needed. Few studies have been conducted addressing this (Garcia-Menendez et al. 2013; Rorig et al. 2013). There have been comprehensive fuel, fire behavior and smoke measurements on prescribed fires (e.g. RxCADRE - Ottmar et al. 2016). However, these data have not yet been used to evaluate and improve smoke modeling systems, especially in terms of meteorological conditions.

The relationship between winds, fire behavior, and smoke dispersion is especially difficult in areas of complex terrain. Wind speed and direction are affected by topography, and vegetation moisture can change at time scales of hours, minutes, and even seconds (Andrews 2012). Additionally, complex topography can directly affect fire behavior and smoke transport (Hardy et al. 2001; Agee et al. 2010). Understanding how fuels, fire, topography and wind patterns interact for a particular region can improve the ability to predict how and where smoke will disperse.

The Deschutes National Forest (DNF), located in the Cascade Mountains west of Bend, Oregon, uses prescribed fire in the Wildland-Urban Interface (WUI) to meet management objectives while protecting the public’s quality of life in nearby communities. A major constraint is the potential for smoke from prescribed fires to reach nearby communities. The state of Oregon Administrative Rules include regulations regarding smoke intrusions, where a “smoke intrusion” is defined as the verified entrance of ground-level smoke from prescribed fires into designated smoke sensitive areas (2007). These are characterized by the one-hour average PM2.5 concentration above the previous three-hour average PM2.5 concentration.

The U.S. Forest Service AirFire research team collaborated with the DNF on a field measurement and modeling project to study the complex interaction of fuels, fire, topography and wind patterns to better understand smoke intrusions from prescribed fires. During the study period, smoke from six prescribed fires intruded into Bend, OR, with 1-hr PM2.5 concentrations ranging from 11 g/m3 to 245 g/m3. This study is unique because few case studies synthesize meteorological conditions, smoke dispersion modeling, and ground-based observations, especially for prescribed fires. Wildfires tend to be more well-studied and large-scale evaluation of air quality prediction systems that include wildfire emissions have been done (e.g. Strand et al. 2012).

The overall goal for this study was to characterize the conditions under which smoke intrusions occur so that smoke intrusions could be better predicted and avoided in the future. To investigate the spatial variability in meteorological conditions and analyze smoke dispersion in the area surrounding Bend, OR, we deployed portable PM2.5 and meteorological measurement stations for approximately nine months during the prescribed fire seasons. The objectives of this study were to: 1) characterize the frequency of acceptable meteorological conditions for prescribed fire; 2) describe seasonal wind patterns and identify circumstances that may avoid smoke intrusions into populated areas; and 3) evaluate existing meteorological and smoke dispersion modeling performance during smoke intrusions. This is one of the few studies gathering a comprehensive dataset of meteorological measurements, PM2.5 measurements, and documented burn information. The results of this study will improve our understanding of smoke dispersion modeling, support the planning of prescribed fires, and ultimately lead to better predictions and fewer smoke intrusions.

# Study Area

The study area was located in Deschutes County, OR, near the cities of Sisters, Bend, and Sunriver (Figure 1). The DNF lies approximately six km to the west of Bend. Annual precipitation in Bend is about 288 mm/year, with the majority of precipitation occurring between November and February (Western Regional Climate Center; http://www.wrcc.dri.edu). The climate of Deschutes County is classified as moist subtropical mid-latitude climate with a dry and warm summer season according to the Köppen climate classification. The vegetation in the area consists of Douglas-fir, Ponderosa pine, Lodgepole pine, juniper, and savanna sagebrush. Prescribed fire in the DNF is typically conducted in the spring (April – June) and fall (September – November) months, and wildfires are a concern during the summer months (July, August, and into September).

Methods

# Acceptable Burn Days

To determine how frequently land managers can expect conditions that are favorable for prescribed fire, we compiled the number of days fuel and meteorological parameters meet required conditions (Table 1). The days were identified by data measured and calculated from the Remote Automated Weather Stations (RAWS) in the area, and include temperature, RH, wind speed, and 1-hr, 10-hr, and 100-hr dead fuel moistures. We used Fire Family Plus, a software system for summarizing and analyzing historical daily fire weather observations (Bradshaw and McCormick 2000), to identify days meeting the acceptable burn conditions. We used data from three RAWS located in the study area and maintained by the DNF and the Western Regional Climate Center (WRCC). These are permanently located stations with weather sensors located approximately 6 m above ground level and are typically placed in locations suitable for monitoring fire danger.

Daily fire weather observations (13:00 PST) from the Tumalo Ridge RAWS (seven km west of Bend at an elevation of 1220 meters; Bend elevation is 1105 meters), Lava Butte RAWS (15 km south of town at an elevation of 1344 meters), and Round Mountain RAWS (47 km southwest of Bend at an elevation of 1798 meters) were used to determine if burning would be within prescription for the days in the period from 2006-2015 (see Table 2 for a list of instrument locations and details).

Seasonal and Diurnal Wind Analysis

In addition to determining the frequency of days in prescription, we also generated seasonal wind roses, both for day and night, to better understand wind patterns in the study area (Appendix 1). We followed the methodology used by the WRCC to define “day” and “night” such that time windows for “daytime” winds included the interval from 11:00 am – 18:00 pm PST and nighttime windows included the interval from 01:00 am – 07:00 am PST. These time periods capture the general wind patterns during the day and night and attempt to reduce the inclusion of transitions associated with sunrise and sunset. Additionally, the times generally cover daytime and nighttime hours throughout the year and minimize the difference between winter and summer.

Seasons were defined by meteorological seasons with the twelve calendar months into four three-month periods. Winter includes the months of December, January, and February; spring includes March, April, and May; Summer includes June, July, and August; and autumn includes September, October, and November. Although the DNF prescribed fire season occurs in autumn and spring, some prescribed fires took place in early June, which would be categorized in this manner as summer. This grouping aligned calendar dates more closely with temperatures during that period and allowed easier comparison of weather patterns across seasons.

# Smoldering Fuel Consumption Measurements

Ottmar et al. (2014) performed post-fire fuel consumption measurements of stumps, logs and basal accumulations (litter and duff deposits at the base of standing trees) at two sites in the DNF (Figure 1) – the West Bend unit (located less than five km WSW of downtown Bend) and the Glaze Meadow unit (approximately 40 km NNW of downtown Bend). These measurements provide estimates of fuel loadings and consumption, which are critical for smoke dispersion modeling. Due to the overnight timing of smoke dispersal, smoldering combustion of downed woody debris was thought to contribute to a smoke intrusion in spring 2014. Because this was a retrospective study, estimates of the timing and duration of smoldering combustion could not be determined.

# Weather and Smoke Measurement Stations

WatchDog Weather Stations (Spectrum, Inc.) were deployed at six sites in 2014 and four sites in 2015 (Figure 1, Table 2) to supplement permanent stations. The portable weather stations collected temperature, precipitation, relative humidity (RH), wind speed, wind direction, wind gust speed, wind gust direction, and dew point at 10- or 15-minute intervals. E-samplers (Met One Instruments, Inc.) were deployed at five sites in 2014 and three sites in 2015. These monitors collect PM2.5 concentration data via light scattering in addition to temperature, RH, wind speed, and wind direction. The E-sampler calculates PM2.5 concentration by applying a calibration to the measured light scattering. This method of observing particulate matter is subject to uncertainty as light scattering may be affected by chemical composition of biomass, the fraction of light absorbing black and brown carbon, and size distribution of particles (Martinsson et al. 2015; Holder et al. 2016; Liu et al. 2017).

The portable sensors deployed for this study were placed at 1.5 meters to two meters above ground level (AGL). Four of the E-samplers recorded 1-hr averages and one recorded 10-min averages in 2014. The three deployed in 2015 used 15-min averages. The State of Oregon operates two permanent nephelometers, which measure light scattering due to particulate matter in the atmosphere which is related to PM2.5 concentration, in the DNF: one at the Sisters, Oregon District Ranger Station in Sisters, and the other at the Bend Pump Station.

# Smoke Dispersion Modeling

The BlueSky smoke modeling framework (Larkin et al. 2010) was used to model the smoke intrusions. BlueSky links together datasets and models of fire location and growth, fuel loadings and consumption, emissions from consumed fuels, plume rise, and smoke dispersion. The dispersion model requires meteorological model output to predict movement and concentration of smoke. For the current study, we used actual fire location and size for each of the prescribed fires. This information was obtained from the intrusion reports prepared by the DNF District Office who were responsible for the burns and contained the dates, times, locations, sizes, and fuel loadings (Table 3).

Six smoke intrusions impacted the city of Bend, OR over the fall of 2014 and spring of 2015 prescribed fire seasons (Table 4). The intrusion reports did not specify fuel loadings by category (1-hr, 10-hr, shrub, etc.), so fuel loadings were obtained from the Fuel Characterization Classification System (FCCS) mapped at a 1-km resolution (Prichard et al. 2013). FCCS fuel models and total loadings used in the model runs are given in Table 3.

A three-dimensional wind field from the Weather Research and Forecast (WRF) model (Michalakes et al. 2001; Skamarock et al. 2005) was used in BlueSky, and the Hybrid Single Particle Langrangian Integrated Trajectory (HYSPLIT) model was used for the dispersion simulations (Draxler and Hess 1998; Stein et al. 2015) (Table 5). The spatial and temporal resolutions of the BlueSky runs are determined by the meteorological model. In this case, we used the hourly 4-km resolution WRF model provided by the University of Washington Department of Atmospheric Sciences (Mass et al. 2003). Additionally, we had available a 1-km resolution meteorological model from the National Centers for Environmental Prediction (NCEP) North American (NAM) weather model (Rogers et al. 2009) for the October 4-5, 2014 smoke intrusion period. Smoke modeling with these data provide hourly predictions of near-surface PM2.5 concentrations. The suite of portable meteorological and PM2.5 monitors were not deployed for the October 2014 episode, but smoke dispersion modeling was possible and the modeled meteorological wind fields were compared with those obtained from RAWS stations. Modeled PM2.5 values were compared with PM2.5 measurements at the Bend Pump Station.

Results

Acceptable Burn Days

Over the 2006-2015 time period, Tumalo Ridge had 259 burn days, Lava Butte had 264 burn days, and Round Mountain had 280 burn days within acceptable meteorological parameters for prescribed fire (Figure 2). Therefore, an average of 26-28 burn days occurred each year. Considerable data gaps existed in the RAWS data during mostly the winter months, so those data are probably biased low during these times. Greater confidence is placed in the spring, summer and fall months of data (highlighted by box around those months in Figure 2).

Seasonal and Diurnal Wind Analysis

We used wind data from the Tumalo Ridge RAWS (the station closest to Bend) to estimate the frequency that daytime and nighttime winds were from a direction that would carry smoke away from Bend. We assessed how often northwesterly through northeasterly winds occurred during the day (to transport smoke away from Bend) and how often southeasterly to southwesterly winds occur during the nighttime (to determine if nighttime drainage flows are responsible for the smoke intrusions) (Table 6). The most suitable wind conditions in this region occur when north winds occur during the day and south winds do not occur at night. Nighttime southerly winds occur 69% – 80% of the time, making acceptable wind conditions a somewhat rare occurrence (5% - 13%).

The Tumalo Ridge stations showed strong S – WSW flows at night for all seasons while Lava Butte nighttime wind was SSW – WSW (Appendix A). During the day Lava Butte also had a similar pattern of SW flows with some northerly flows as well. Round Mountain was different in that it had a NW pattern, with some flows from all directions as well. This station is located at a higher elevation (1798 m) and was the furthest removed (47 km) from Bend. Seasonal analysis shows that these wind conditions occur 77% of the time annually.

# Fuel Consumption Estimations

Accurate estimates of fuel loadings and types are necessary for consumption and emissions predictions. Data on pre-fire fuel information for the three targeted fuelbed components were unavailable, so postburn data were collected approximately two months later to reconstruct the potential contribution of stumps, logs and basal accumulations to smoldering combustion and to smoke production (Ottmar et al. 2014). Total maximum smoldering fuel component consumption was estimated at 3094 kg/ha in West Bend and 17553 kg/ha in Meadow Glade with over 50% of that consumption from smoldering stumps. West Bend had minimal smoldering of logs (247 kg/ha) while Meadow Glade had 6882 kg/ha. Consumption of basal accumulation was similar at 695 kg/ha and 852 kg/ha at West Bend and Meadow Glade respectively. This information about the smoldering combustion components was used in the smoke modeling to improve predicted PM2.5 concentrations from the intrusions analyzed in this work.

Smoke Intrusions

Smoke from prescribed fires intruded into Bend on one occasion in October 2014 and five occasions in May thru June 2015 (Table 4). The May 4, 2015 intrusion was the shortest duration and lowest concentration and occurred during the daytime hours. The other five intrusions occurred in the evening, over-night, and early morning hours, with 1-hr PM2.5 concentrations up to 245 μg/m3. Here we discuss the measured meteorological conditions contributing to these intrusions, a graphical and statistical analysis of the modeled wind field from the 4-km WRF meteorological prediction system for all six intrusions and the modeled 1-km resolution wind field from the NWS for the October 2014 intrusion.

Three prescribed fires were ignited on October 4, 2014 approximately 44 km SSW of Bend (Figure 1). Each burn unit was between 18 and 20 ha. Ignition occurred between 1100 PDT and 1400 PDT and smoke was initially carried away from town. Overnight, however, conditions changed and smoke was transported into town. Elevated PM2.5 values registered an intrusion starting at 0200 PDT October 5 and dissipating by 1200 PDT. A maximum 1-hr PM2.5 concentration of 96 μg/m3 was recorded at the Bend Pump Station at 0300 PDT, with a second peak of 94 μg/m3 at 0900 PDT.(Figure 3).

A 1-km resolution NAM meteorological model domain (36-hour forecast) was available from the NWS in addition to the 4-km WRF meteorological domain. Comparison of winds and smoke dispersion was undertaken with the two resolutions and compared to the three available RAWS sites (Tumalo Ridge, Lava Butte, and Round Mountain). Wind direction mean errors for both 1-km (left side of Figure 4) and 4-km (right side of Figure 4) resolutions, day and night, are shown for the 36-hour period. For the 1-km resolution domain, daytime mean errors ranged from 45 to 80 degrees, while nighttime mean errors ranged from 20 to 80 degrees. For the 4km resolution domain, daytime mean errors ranged from 38 to 60 degrees and nighttime mean errors ranged from less than 10 to greater than 80 degrees.

The BlueSky smoke model simulations using both the 1-km resolution NWS NAM and the 4-km WRF show smoke transported down the drainage from the SSW into Bend (Figures 5a and 5b respectively) with the plume arriving at 0300 PDT in agreement with the measured data. Predicted concentrations were lower than measured (approximately 10-15 μg/m3 for the 1-km NAM output and less than 1 μg/m3 for the 4-km WRF output; Figure 3). This was probably due to BlueSky not fully capturing the smoldering of basal accumulations and large woody debris. The smoke simulation using the 1-km NAM showed a well-defined plume transporting along the drainage. The lower resolution 4-km WRF simulation carried some smoke towards Bend overnight, but for the most part the model results show the plume east of town.

A 46 ha prescribed fire was ignited at 0930 PDT May 4, 2015 approximately 55 km southwest of Bend, Oregon (Figure 1). Smoke was transported into Bend within 3.5 hours of ignition and a maximum concentration of 13 μg/m3 was recorded at the Bend Pump Station nephelometer. Concentrations were elevated for approximately two hours. Before ignition on May 4, the Tumalo Ridge RAWS measured winds from the north, however by the time of ignition the winds were from the south. Winds were steady from the WSW during the intrusion period, indicating that smoke could be transported into town. Conversely, the Round Mountain RAWS, which was the closest wind monitor to the burn, had WNW winds at the time of ignition and throughout the afternoon, suggesting the wind should have carried the smoke away from Bend. Other weather stations located along the Hwy 97 corridor between Bend and the burn measured predominantly southerly winds (see the first shaded area in Figure 6).

BlueSky smoke modeling results simulated the timing of the transport of the smoke into Bend (Figure 7). However, the modeled BlueSky 1-hr PM2.5 concentrations at the Bend Pump Station were an order of magnitude less than the concentrations measured by the nephelometer, with concentrations of 0.05 to 0.47 μg/m3 predicted between 1300 and 1500 PDT compared to the observed results of 4.8 to 12.6 μg/m3. Figure 7 shows how the modeled plume centerline was pushed to the south of town, and thus only the plume fringes were predicted to impact Bend. This can also be seen in Figure 6 (first intrusion gray-shaded area) which shows measured and modeled wind speed and wind direction data at Cascade Middle School. Modeled winds were WSW while measured winds were S-SW. Figure 7 shows PM2.5 concentrations from the model run using the increased fuel loadings. Also, note that we are only simulating primary PM2.5 emissions from the fires and not taking into account secondary formation or other potential sources.

The five nighttime and early morning intrusions in 2015 all exhibited similar characteristics. The prescribed fires were located 6-km to 10-km SSW of Bend. During the day NE - NW winds transported smoke away from town. Overnight, winds decreased and became SW with PM2.5 concentrations becoming elevated in Bend. BlueSky 4-km resolution simulations weakly simulated smoke transport into town for the May 5 intrusion around midnight but did not bring smoke into town during the intrusion period of 0600-0800 PDT. BlueSky smoke model simulations also failed to bring smoke into town for the May 28, June 5 and June 6 intrusions.

Figures 6, 8, and 9 illustrate the measured wind directions and wind speeds at the meteorological station in town, measured at Cascade Middle School, for each of the intrusions. During the overnight intrusion periods (see the second shaded area of Figure 6 and all the shaded areas in Figures 8 and 9) the 4-km WRF modeled winds remained from the NW while measured winds were from the SSW. Mean wind direction errors ranged from 89 – 108 degrees at night at this location (Appendix 2). Figure 10 shows box plots of the day and night wind direction mean error values for all the intrusion periods. In general, mean wind direction errors were greater at night than during the day. The modeled wind speeds were generally biased high within 1-3 m/s of the observed values and calm winds registered greater than 50% of the time at four out of the nine stations (Table 7), largely due to calm winds overnight (Appendix 2).

Discussion

Acceptable Burn Days

Given the tendency for prescribed fires to be planned in the spring and autumn, missing observations over winter in our burn day analysis may not have much effect on planning. Additionally, many of the days within prescription occur during the summer months (2-7 prescription days/month), which coincide with wildfire season, when prescribed fire is typically not used. Fire behavior models and weather conditions are used to estimate fire behavior and reduce the risk of escape, minimize the cost of control, and reduce impacts on the surrounding environment (Albini 1976). Prescribed fire planning must take into account expected weather and acceptable fire behavior as variations in fuels and weather may cause unpredictable fire behavior that poses risks to fire suppression activates (Rothermel 1972). Fuel moistures must be low enough so the fuels will ignite and carry the fire, but not so low that the fire could get out of control. Likewise, the air temperature and relative humidity must be warm and dry enough so the fire will carry, but again not so hot and dry that the fire could burn uncontrollably, as happened in New Mexico with the Cerro Grande Fire near Los Alamos National Lab in 2000 (Paxon 2000). Our data show that there are relatively few days (1-6 prescription days/month in the Spring and 2-4 prescription days/month in the Fall) that meet the desired conditions for ignition.

Seasonal and Diurnal Wind Analysis

Wind speed and direction are critical for determining where smoke will go and the need to keep smoke away from populated areas further decreases the number of available burn days. We concluded that in the Bend, OR region, even when burn conditions may be favorable for desired fire behavior, wind directions may not be acceptable due to smoke intrusion concerns. Our results indicated that many days that would be acceptable for fire ignition would likely result in smoke intrusions into nearby communities.

Fuel moisture and winds are two of the most important factors affecting wildland fire behavior, and winds can be highly variable and unpredictable (Buck and Schroeder 1970). Wind speed and direction are affected by topography and vegetation and can change at time scales of hours, minutes, and even seconds (Andrews 2012). Topography can directly affect fire behavior through the channeling of winds, which are typically strong along major streams incised through mountain valleys (Agee et al. 2010). As cold air from radiation cooling at night drops into mountain valleys, they cause downslope winds to form (Hardy et al. 2001).  Less turbulence at night further promotes winds that follow terrain.  These downslope winds generally occur from sunset to sunrise (Hardy et al. 2001).

The analysis of wind patterns in the area revealed the difficulty of planning prescribed fires on moderate weather days that met the parameters for burning while also being amenable dispersing smoke away from nearby communities. While daytime wind flows may be in an acceptable direction, there were patterns of shifting wind direction overnight that could lead to smoke intrusions. Additionally, when fuels from prescribed fires continue to burn overnight, combustion may take place in the smoldering phase, which produces large amounts of particulate matter– more than double the particulate emissions than the flaming phase (Hardy et al. 2001; Urbanski 2014). Smoldering is more common in fuel types including duff and rotten logs. Additionally, there is often not enough heat generated by the fire during the smoldering phase to produce a convection column, and the result is that smoke and pollutants stay near the ground and concentrate in valley bottoms (Hardy et al. 2001).

Smoke Intrusions

The nighttime smoke intrusion modeling from the nighttime smoke intrusions in 2014 demonstrated how shifts in wind directions, especially those that occur down valley at night, led to smoke intrusions into nearby communities in spite of satisfactory conditions at the time of ignition.  The use of higher resolution meteorological models, which can better resolve complex terrain features, improved smoke dispersion predictions. The high resolution meteorological model output showed smoke dispersion was strongly affected by the local topographic relief. While this may not always be the case, higher resolutions have been shown to provide improved results when compared with coarser resolutions in modeling fire danger indices (Hoadley et al. 2006).

We found consistently lower smoke concentrations predicted by the models than observed with deployed monitors. Ottmar et al. (2014) identified smoldering consumption of duff, stumps and basal accumulations as likely contributing additional smoke in the atmosphere. For modeling the daytime smoke intrusions from 2015, increasing the duff depth from two inches to five inches in the model approximately doubled the pre-burn fuel load, with most of that in the smoldering phase causing it to be released close to the ground. This improved the BlueSky’s predicted concentrations, although the main plume was still simulated to miss town because predicted winds did not change.

Comparing the predicted and observed winds and particulate matter across the intrusions indicated conditions that were common to all but one of the intrusions. Most cases occurred during the late night and early morning hours, when winds were light or calm, and smoke movement was driven by terrain-induced down-drainage flows. The unique case was on May 4 2015, when daytime winds carried smoke into Bend in the early afternoon, two hours after ignition. In all cases the prescribed fires were located southwest of town and smoke was transported into town by southwest winds.

The accuracy of smoke dispersion model results varied by intrusion. In some cases, the model results showed smoke transport into Bend close to the time indicated by the observations (the daytime intrusions of May 4, and the nighttime intrusions of May 5 - 6 2015, and October 5, 2014). The model results for the other cases (May 28-29 2015, June 5 and 6 2015) showed no smoke transport into Bend predicted. When both the observations and the model showed smoke in Bend, the modeled concentrations were less than observed, sometimes by an order of magnitude or more. This suggests that emissions from smoldering fuels were likely underestimated. Furthermore, dispersion models are only as good as the underlying meteorological model (Garcia-Menendez et al. 2013), and if that model does not accurately represent the winds (such as sub-grid scale drainage winds), the dispersion model will not accurately transplant the smoke. Wind direction mean errors ranged from 14 – 94 degrees with even higher mean errors during the night. The one case where two resolutions were available (October 4 2014, 4km and 1km), the higher resolution model better predicted the location and timing of the smoke intrusion.

Future Research

Improving the characterization of the smoldering fuels and using higher resolution meteorological data both improved the smoke modeling results. Future research is required to include pre-fire fuel loading measurements and to refine the measurement of the consumption of forest fuels during the flaming and smoldering phases of combustion, and the timing and the duration of that consumption. Continued improvement of meteorological models is critical in order to deliver the smoke to the correct location and high resolution model output shows promise for areas of complex terrain. Additionally, accurate assessments of fuel types and loadings are essential for realistic estimates of emissions. Unfortunately, current fuel models do not adequately represent the smoldering fuels that often are responsible for smoke intrusions in this region. For smoke managers, it may no longer be enough to base prescribed fire plans on the total amount of forest fuels, fuel consumption, and total smoke produced on site. Rather, a more detailed understanding of the timing of consumption and smoke production during periods of weak atmospheric dispersal may better help manage downwind smoke effects in communities near the WUI.

Conclusion

This study assessed six smoke intrusion episodes in the autumn of 2014 and spring of 2015 in Bend, OR. On average, there were 26-28 days per year with suitable meteorological conditions for prescribed fire behavior. Wind direction further constrained the predicted number of acceptable burn days due to the potential for smoke dispersal into city centers. Our results demonstrate the utility of dispersion modeling for predicting smoke intrusions. However, considerable errors in wind speed and direction of the meteorological models may produce poor model results and cause mischaracterization of smoke intrusion events. The results of higher resolution meteorological and dispersion models shows their potential for improving the prediction of both timing and location of smoke intrusions. Finally, this study highlights the difficulty of planning and implementing prescribed fires in a region where complex terrain and weather patterns severely limit conditions for smoke dispersal that would avoid health and safety impacts to nearby communities.

Acknowledgements

We thank the USDA Forest Service Region 6 and the Washington and Oregon Offices of the Bureau of Land Management for funding this study. We also would like to thank the many people who made this study possible, including: Deana Wall and everyone at the Deschutes National Forest Supervisors Office, Jinny Reed and others at the USDA Forest Service Sisters, Oregon Regional Office, Bridges Boys Academy, Cascade Academy, Miller Elementary, Cascade Middle School, Tumalo Golf Club, Colin Wills with Arnold Irrigation District, and Jason Schneider with the Sunriver Home Owners Association.

Literature Cited

Agee JK, Edmonds RL, Gara RI. 2010. Forest health and protection. 2nd ed. Long Grove, IL: Waveland Press.

Albini FA. 1976. Estimating wildfire behavior and effects.

Andrews PL. 2012. Modeling wind adjustment factor and midflame wind speed for Rothermel’s surface fire spread model. Gen Tech Rep RMRS-GTR-266 Fort Collins, CO US Dep Agric For Serv Rocky Mt Res Station 39 p. 266.

Bowman DMJS, Balch JK, Artaxo P, Bond WJ, Carlson JM, Cochrane MA, D’Antonio CM, DeFries RS, Doyle JC, Harrison SP. 2009. Fire in the Earth system. Science (80- ). 324(5926):481–484.

Bradshaw L, McCormick E. 2000. FireFamily Plus user’s guide, version 2.0. Gen Tech Rep RMRS-GTR-67 Ogden, UT US Dep Agric For Serv Rocky Mt Res Station. 67.

Buck CC, Schroeder MJ. 1970. Fire weather: a guide for application of meteorological information to forest fire control operations. Buck CC, editor. Washington, D.C.: U.S. Dept. of Agriculture, Forest Service.

Characterization of smoke incidents or intrusions. 2007. Oregon Administrative Rules, Division 48, Chapter 629.

Covington WW, Fule PZ, Moore MM, Hart SC, Kolb TE, Mast JN, Sackett SS, Wagner MR. 1997. Restoring ecosystem health in ponderosa pine forests of the Southwest. J For. 95(4):23.

Covington WW, Moore MM. 1994. Southwestern ponderosa forest structure: changes since Euro-American settlement. J For. 92(1):39–47.

Delfino RJ, Staimer N, Tjoa T, Gillen DL, Polidori A, Arhami M, Kleinman MT, Vaziri ND, Longhurst J, Sioutas C. 2009. Air pollution exposures and circulating biomarkers of effect in a susceptible population: clues to potential causal component mixtures and mechanisms. Environ Health Perspect. 117(8):1232.

Draxler RR, Hess GD. 1998. An overview of the HYSPLIT_4 modelling system for trajectories. Aust Meteorol Mag. 47(4):295–308.

Garcia-Menendez F, Hu Y, Odman MT. 2013. Simulating smoke transport from wildland fires with a regional‐scale air quality model: Sensitivity to uncertain wind fields. J Geophys Res Atmos. 118(12):6493–6504.

Graham RT, McCaffrey S, Jain TB. 2004. Science basis for changing forest structure to modify wildfire behavior and severity. Bark Beetles, Fuels, Fire Bibliogr.:167.

Hardy CC, Ottmar RD, Peterson JL, Core JE, Seamon P. 2001. Smoke management guide for prescribed and wildland fire: 2001 edition. PMS 420-2 NFES 1279 Boise, ID Natl Wildfire Coodination Group 226 p.

Hessburg PF, Agee JK, Franklin JF. 2005. Dry forests and wildland fires of the inland Northwest USA: contrasting the landscape ecology of the pre-settlement and modern eras. For Ecol Manage. 211(1–2):117–139.

Hoadley JL, Rorig ML, Bradshaw L, Ferguson SA, Westrick KJ, Goodrick SL, Werth P. 2006. Evaluation of MM5 model resolution when applied to prediction of National Fire Danger Rating indexes. Int J Wildl Fire. 15(2):147–154.

Holder AL, Hagler GSW, Aurell J, Hays MD, Gullett BK. 2016. Particulate matter and black carbon optical properties and emission factors from prescribed fires in the southeastern United States. J Geophys Res Atmos. 121(7):3465–3483.

Kochi I, Donovan GH, Champ PA, Loomis JB. 2010. The economic cost of adverse health effects from wildfire-smoke exposure: a review. Int J Wildl Fire. 19(7):803–817.

Larkin NK, O’Neill SM, Solomon R, Raffuse S, Strand T, Sullivan DC, Krull C, Rorig M, Peterson J, Ferguson SA. 2010. The BlueSky smoke modeling framework. Int J Wildl Fire. 18(8):906–920.

Liu X, Huey LG, Yokelson RJ, Selimovic V, Simpson IJ, Müller M, Jimenez JL, Campuzano‐Jost P, Beyersdorf AJ, Blake DR. 2017. Airborne measurements of western US wildfire emissions: Comparison with prescribed burning and air quality implications. J Geophys Res Atmos. 122(11):6108–6129.

Martinsson J, Eriksson AC, Nielsen IE, Malmborg VB, Ahlberg E, Andersen C, Lindgren R, Nystrom R, Nordin EZ, Brune WH. 2015. Impacts of combustion conditions and photochemical processing on the light absorption of biomass combustion aerosol. Environ Sci Technol. 49(24):14663–14671.

Mass CF, Albright M, Ovens D, Steed R, MacIver M, Grimit E, Eckel T, Lamb B, Vaughan J, Westrick K. 2003. Regional environmental prediction over the Pacific Northwest. Bull Am Meteorol Soc. 84(10):1353–1366.

Michalakes J, Chen S, Dudhia J, Hart L, Klemp J, Middlecoff J, Skamarock W. 2001. Development of a next-generation regional weather research and forecast model. In: Developments in Teracomputing. World Scientific. p. 269–276.

Mitchell SR, Harmon ME, O’connell KEB. 2009. Forest fuel reduction alters fire severity and long‐term carbon storage in three Pacific Northwest ecosystems. Ecol Appl. 19(3):643–655.

Miyanishi K. 2001. Forest Fires: Behavior and Ecological Effects. Elsevier.

Morgan G, Sheppeard V, Khalaj B, Ayyar A, Lincoln D, Jalaludin B, Beard J, Corbett S, Lumley T. 2010. Effects of bushfire smoke on daily mortality and hospital admissions in Sydney, Australia. Epidemiology.:47–55.

Ottmar R, Restiano J, Vihnanek R, Burcke C. 2014. Final Report: Identifying fuelbed components that contribute to smoldering consumption to improve smoke management decisions for prescribed fire application east of the Cascades in Oregon and Washington. Seattle, WA.

Ottmar RD, Hiers JK, Butler BW, Clements CB, Dickinson MB, Hudak AT, O’Brien JJ, Potter BE, Rowell EM, Strand TM. 2016. Measurements, datasets and preliminary results from the RxCADRE project–2008, 2011 and 2012. Int J Wildl Fire. 25(1):1–9.

Paxon J. 2000. Remember Los Alamos”: The Cerro Grande Fire. Fire Manag Today. 60:9–14.

Prichard SJ, Andreu A, Eagle P, Swedin K, Sandberg D V, Ottmar RD, Eberhardt E. 2013. Fuel characteristic classification system version 3.0: Technical documentation. USDA For Serv - Gen Tech Rep PNW-GTR.(887):1–79.

Rappold AG, Stone SL, Cascio WE, Neas LM, Kilaru VJ, Carraway MS, Szykman JJ, Ising A, Cleve WE, Meredith JT, et al. 2011. Peat bog wildfire smoke exposure in rural North Carolina is associated with cardiopulmonary emergency department visits assessed through syndromic surveillance.(Research)(Report). Environ Health Perspect. 119(10):1415. doi:10.1289/ehp.1003206.

Rogers E, DiMego G, Black T, Ek M, Ferrier B, Gayno G, Janjic Z, Lin Y, Pyle M, Wong V, et al. 2009. The NCEP North American Mesoscale Modeling System: Recent changes and Future Plans. In: 23rd Conference on Weather Analysis and Forecasting / 19th Conference on Numerical Weather Prediction. Omaha, NE.

Rorig M, Solomon R, Krull C, Peterson J, Ruthford J, Potter B. 2013. Analysis of meteorological conditions for the Yakima Smoke Intrusion case study, 28 September 2009. USDA For Serv - Res Pap PNW-RP.(597):1–32.

Rothermel RC. 1972. A mathematical model for predicting fire spread in wildland fuels.

Skamarock W, Klemp J, Dudhia J, Gill D, Barker D, Wang W. 2005. A Description of the Advanced Research WRF Version 2. Library UCFAR :National C for AR :NCAR, editor. doi:10.5065/D6DZ069T.

Stein AF, Draxler RR, Rolph GD, Stunder BJB, Cohen MD, Ngan F. 2015. NOAA’S HYSPLIT ATMOSPHERIC TRANSPORT AND DISPERSION MODELING SYSTEM. Bull Am Meteorol Soc. 96(12):2059–2078. doi:10.1175/BAMS-D-14-00110.1.

Strand TM, Larkin N, Craig K, Raffuse S, Sullivan D, Solomon R, Rorig M, Wheeler N, Pryden D. 2012. Analyses of BlueSky Gateway PM2.5 predictions during the 2007 southern and 2008 northern California fires. J Geophys Res. 117. doi:10.1029/2012JD017627.

Urbanski S. 2014. Wildland fire emissions, carbon, and climate: Emission factors. For Ecol Manage. 317(C):51–60. doi:10.1016/j.foreco.2013.05.045.

Table 1. Prescribed fire prescription parameters for fires conducted on the Deschutes National Forest.

| Parameter | Low | High | Minimal Acceptable Moisture Parameters |
| --- | --- | --- | --- |
| Air Temperature (F) | 40 | 80 | - |
| Relative Humidity (%) | 20 | 40 | - |
| Mid-flame Wind Speed (mph) | 0 | 8 | - |
| 1-hr Fuel Moisture (%) | 5 | 10 | 5 |
| 10-hr Fuel Moisture (%) | 6 | 12 | 6 |
| 100-hr Fuel Moisture (%) | 7 | 14 | 7 |
| 1000-hr Fuel Moisture (%) | - | - | 15 |
| Live Fuel Moisture (%) | - | - | 30 |

Table 2. Meteorological stations and smoke monitor locations for Spring 2015. Locations are listed from North to South. WX = Watchdog meteorological measurement station (wind speed, wind direction). Smoke = MetOne Inc. E-Sampler measuring PM2.5 concentrations and wind data (wind speed, wind direction). RAWS = Remote automated weather station measuring wind speed and wind direction. Nephelometer = Radiance Research M903.

| Station | Latitude | Longitude | Elevation | Measurement Type | Relationship to Bend, OR |
| --- | --- | --- | --- | --- | --- |
|  | (deg) | (deg) | (m) |  | (km, direction) |
| Sisters Ranger Station | 44.2925 | -121.5552 | 975 | WX | 32 km, NNW |
| Sisters Ranger Station | 44.2925 | -121.5552 | 975 | Nephelometer | 32 km, NNW |
| Bridges Boys Academy | 44.2274 | -121.5212 | 1079 | WX | 25 km, NW |
| Cascade Academy | 44.1327 | -121.3323 | 988 | WX | 8 km, NNW |
| Tumalo Ridge | 44.0494 | -121.4003 | 1220 | RAWS | 7 km, WSW |
| Bend Pump Station |  |  |  | Nephelometer | In Town |
| Miller Elementary | 44.0543 | -121.3692 | 1167 | WX | 5 km, W |
| Cascade Middle School | 44.0370 | -121.3397 | 1145 | Smoke | 4 km, SW |
| Lava Butte | 43.93 | -121.33 | 1344 | RAWS | 15 km, S |
| Sunriver | 43.9033 | -121.4329 | 1269 | Smoke | 20 km, SSW |
| Round Mountain | 43.6739 | -121.7167 | 1798 | RAWS | 47 km SW |

Table 3. Prescribed fires responsible for smoke intrusions into Bend, Oregon for Fall 2014 and Spring 2015. Vegetation types derived from FCCS were used for Bluesky runs.

| Date | Latitude | Longitude | Elevation | Size | Pre-fire fuel loading | Vegetation type | Ignition Start |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | (deg) | (deg) | (m) | (ha) | (kg/ha) |  | (PDT) |
| 10/04/14 | 43.7250 | -121.6323 | 1301 | 18.2 | 101482 | Lodgepole Pine | 1357 |
| 10/04/14 | 43.6960 | -121.6529 | 1305 | 19.0 | 44498 | Pacific Ponderosa Pine – Douglas-fir | 1114 |
| 10/04/14 | 43.7105 | -121.6329 | 1302 | 20.3 | 44498 | Pacific Ponderosa Pine – Douglas-fir | 1130 |
| 05/04/15 * | 43.6571 | -121.8360 | 1525 | 46.2 | 182990 | Lodgepole Pine | 0930 |
| 05/05/15 | 43.9611 | -121.3339 | 1266 | 4.9 | 30734 | Ponderosa Pine Savanna | 1045 |
| 05/28/15 | 44.0242 | -121.3839 | 1312 | 27.9 | 21005 | Western Juniper/ Sagebrush Savanna | 1125 |
| 06/05/15 | 44.0423 | -121.3975 | 1220 | 49.4 | 21005 | Western Juniper/ Sagebrush Savanna | 1100 |
| 06/06/15 | 44.0136 | -121.3975 | 1234 | 55.9 | 21005 | Western Juniper/ Sagebrush Savanna | 1000 |

* 05/04/15 fuels customized from FCCS #22 by increasing duff depth from 2 to 5 inches

Table 4. Summary of smoke intrusion episodes into Bend, Oregon for Fall of 2014 and Spring of 2015.

| Burn Date | Intrusion Start | Intrusion Duration | Maximum 1-hr PM2.5 | Maximum 24-hr PM2.5 | Relationship to Bend, OR |
| --- | --- | --- | --- | --- | --- |
|  | (PDT) | (hr) | (g/m3) | (g/m3) | (km, direction) |  |
| 10/04/14 | 0300 10/05/14 | 10 | 96 | 26 | 45 km, SSW |
| 10/04/14 |  |  |  |  | 49 km, SSW |
| 10/04/14 |  |  |  |  | 47 km, SSW |
| 05/04/15 | 1300 | 2 | 13 | 5 | 60 km, WSW |
| 05/05/15 | 0700 05/06/15 | 1 | 11 | 2.3 | 11 km, SSW |
| 05/28/15 | 0100 05/29/15 | 7 | 181 | 27 | 7 km, SW |
| 06/05/15 | 2200 | 12 | 130 | 25 | 7 km, WSW |
| 06/06/15 | 0000 06/07/15 | 10 | 245 | 38 | 9 km, SW |

Table 5. The BlueSky Smoke Modeling Framework configuration used for modeling smoke production and transport from the prescribed fires.

| BlueSky Framework | Version 3.1.5 |
| --- | --- |
| Meteorological model | WRF 3.1.1 (4-km, 1-hr intervals), NAM (1-km, 1-hr intervals) |
| Fuel loadings | Fuel Characteristic Classification System (FCCS) |
| Consumption model | CONSUME Version 3 |
| Emissions model | FEPS Version 1 |
| Dispersion model | HYSPLIT Version 4.9 |

Table 6. Percent days when nighttime south winds and daytime north winds occur at the Tumalo Ridge RAWS from 2006 – 2015. Ideal wind conditions are when north winds occur during the day and south winds do not occur at night. “Annual” analysis takes into account all days of the year. “Annual Burn” analysis takes into account only days that meet the prescribed fire prescription window parameters. Similarly for Spring and Fall.

|  | Night South Wind Yes | Night South Wind No | Day North Wind Yes | Day North Wind No | “Ideal” Wind Conditions |
| --- | --- | --- | --- | --- | --- |
| Annual | 77% | 15% | 36% | 45% | 7% |
| Annual Burn | 75% | 17% | 46% | 38% | 8% |
| Spring | 77% | 12% | 45% | 36% | 7% |
| Spring Burn | 69% | 21% | 50% | 36% | 13% |
| Fall | 77% | 17% | 35% | 44% | 9% |
| Fall Burn | 80% | 14% | 46% | 38% | 5% |

Table 7. Wind speed (WS) mean bias, wind speed mean error, and wind direction (WD) mean error at the meteorological measurement stations for the 2015 smoke intrusion periods. Model data are from the 4-km resolution Weather Research Forecast model operated by the University of Washington. Station locations are listed north to south.

|  | May 4-6, 2015 |  | May 28-29, 2015 |  | June 5-7, 2015 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Station | WS Bias/ Error | WD Error | WS Bias/ Error | WD Error | WS Bias/ Error | WD Error |
|  | (m/s) | (deg) | (m/s) | (deg) | (m/s) | (deg) |
| Sisters Ranger Station | 2.9/ 3.0 50% calm | 49.3 | 1.7/ 1.7 88% calm | 43.5 | 2.5/ 2.5 79% calm | 47.3 |
| Bridges Boys Academy | 1.3/ 1.5 40% calm | 43.1 | 1.1/ 1.4 63% calm | 48.0 | 1.1/ 1.1 44% calm | 27.2 |
| Cascade Academy | 2.0/ 2.0 67% calm | 43.8 | 1.3/ 1.3 71% calm | 45.4 | 2.8/ 2.8 63% calm | 38.3 |
| Tumalo Ridge RAWS | 0.6/ 1.1 | 42.9 | 0.6/ 0.9 | 58.6 | 0.9/ 1.1 | 40.8 |
| Miller Elementary | 1.6/ 1.8 52% calm | 33.8 | 1.8/ 1.8 79% calm | 70.2 | 2.6/ 2.6 56% calm | 29.7 |
| Cascade Middle School | 0.4/ 0.7 | 59.9 | 0.7/ 1.0 | 85.4 | 0.9/ 1.1 | 74.3 |
| Lava Butte RAWS | 0.7/ 1.4 10% calm | 34.8 | 0.9/ 1.3 33% calm | 59.4 | 2.0/2.0 19% calm | 39.2 |
| Sunriver | -0.6/ 1.0 31% NA | 71.2 | 0.4/ 1.2 | 94.0 | 0.9/ 1.1 | 55.4 |
| Round Mountain RAWS | -0.2/ 1.4 | 13.8 | -0.7/ 1.5 | 59.5 | 0.1/ 1.6 | 62.6 |

Figure 1. Study area in and around Bend, OR, showing locations of co-located smoke and meteorological monitors (smoke icon), weather stations only (cloud icon), prescribed fires (fire icon) and fuel sampling sites (log icon).

Figure 2. Mean number of days weather conditions are within prescription, by month, 2006 - 2015 for Lava Butte, Round Mountain, and Tumalo Ridge RAWS sites. The whiskers represent the maximum for each month over the 10-year period. The box encompasses months when data are complete.

Figure 3. October 4 – 5, 2014, measured PM2.5 concentrations at the Bend Pump Station. Red line indicates time of ignition, and shaded area represents the time of the intrusion.

Figure 4. October 4 - 6, 2014. Daytime (blue) and nighttime (red) mean wind direction error of modeled (1km NAM on the left side; 4-km WRF on the right side) minus measured wind data at the three RAWS.

Figure 5a. BlueSky output using 4km WRF model, valid time 0700 PDT 5 October 2014.

Figure 5b. BlueSky output using 1km NAM model, valid time 0700 PDT 5 October 2014.

Figure 6. Measured and modeled wind direction and wind speed at Cascade Middle School for May 4-6, 2015. Red lines indicate burn ignition times and gray-shaded areas indicate the smoke intrusion periods into Bend, Oregon.

Figure 7. Meteorological and smoke measurement locations in the town of Bend, Oregon and south of town and the burn locations for the May 4-6, 2015 smoke intrusion period. The red contours are the near-surface 1-hr PM2.5 concentrations predicted by the BlueSky smoke modeling framework May 4, 2015 at 1400 PDT.

Figure 8. Measured and modeled wind data at Cascade Middle School for the 5/28/2015 - 5/29/2015 intrusion. Red line indicates burn ignition time and gray-shaded area indicates the smoke intrusion period into Bend.

Figure 9. Measured and modeled wind speed and direction for the June 5-6, 2015 smoke intrusions. Wind data are from Cascade Middle School. Red lines indicate burn ignition times and gray-shaded areas indicate the smoke intrusion periods into Bend.

Figure 10. Box plots showing day and night median (center line), 25th and 75th percentiles (bottom and top of the box, respectively), maximum and minimum values (whiskers), and mean (black box) wind direction errors (modeled – observed) for the 2015 intrusion model runs.

# Appendix A

Hourly weather observations for the years 2006-2015 were collected from Remote Automated Weather Stations (RAWS) at Round Mountain, Tumalo Ridge, and Lava Butte, located in the Deschutes National Forest.  Most RAWS units are owned by wildland fire agencies and placed in locations where they can monitor fire danger (http://raws.fam.nwcg.gov).

To help understand the diurnal weather patterns around Bend, Oregon, we created wind roses from the RAWS observed winds, following the Northwest Coordination Center (NWCC) format. The wind roses show the frequency of winds blowing from a particular direction, with the length of each spoke around the circle representing the frequency of observations from that direction (http://www.wcc.nrcs.usda.gov/climate/windrose.html). Time periods for “daytime” winds include the interval from 1100 – 1800 PST, and nighttime windows include the interval from 0100 – 0700 PST.   These time periods capture the general wind patterns during the day and night and attempt to reduce the inclusion of transitions associated with sunrise and sunset.  Additionally, the times generally cover daytime and nighttime hours throughout the year and minimize the difference between winter and summer.

We used meteorological seasons to categorize the data, grouping the twelve calendar months into four 3-month periods.  Winter includes the months of December, January, and February; spring includes March, April, and May; Summer includes June, July, and August; and autumn includes September, October, and November.  This grouping aligns calendar dates more closely with temperatures during that period, as well as allowing easier comparison of weather patterns from one season to another.  The wind roses for all RAWS and all seasons are shown below.

Figure A1. Locations of RAWS stations closest to Bend, OR.

Figure A2. Wind roses for Tumalo Ridge in the spring during the daytime (left) and nighttime (right).

Figure A3. Wind roses for Tumalo Ridge in the summer during the daytime (left) and nighttime (right).

Figure A4. Wind roses for Tumalo Ridge in the autumn during the daytime (left) and nighttime (right).

Figure A5. Wind roses for Tumalo Ridge in the winter during the daytime (left) and nighttime (right).

Figure A6. Wind roses for Round Mountain in the spring during the daytime (left) and nighttime (right).

Figure A7. Wind roses for Round Mountain in the summer during the daytime (left) and nighttime (right).

Figure A8. Wind roses for Round Mountain in the autumn during the daytime (left) and nighttime (right).

Figure A9. Wind roses for Round Mountain in the winter during the daytime (left) and nighttime (right).

Figure A10. Wind roses for Lava Butte in the spring during the daytime (left) and nighttime (right).

Figure A11. Wind roses for Lava Butte in the summer during the daytime (left) and nighttime (right).

Figure A12. Wind roses for Lava Butte in the autumn during the daytime (left) and nighttime (right).

Figure A13. Wind roses for Lava Butte in the winter during the daytime (left) and nighttime (right).

Appendix B

Table B1. Wind speed bias and error, wind direction error, and percent calm observations for May 4, 2015.

Table B2. Wind speed bias and error, wind direction error, and percent calm observations for day and night on May 5, 2015.

Table B3. Wind speed bias and error, wind direction error, and percent calm observations for day and night on May 28, 2015.

Table B4. Wind speed bias and error, wind direction error, and percent calm observations for day and night on June 5, 2015.

Table 5. Wind speed bias and error, wind direction error, and percent calm observations for day and night on June 6, 2016.

Table 6. Wind speed bias and error, wind direction error, and percent calm observations for day and night on October 4, 2014.